#define BLYNK_TEMPLATE_ID "TMPL3jwFbzJ5J"

#define BLYNK_TEMPLATE_NAME "SMARTMETER2"

#define BLYNK_AUTH_TOKEN "xiFjuKAt8xvHrPaeOibwbRTTfYsjqNX3"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include "EmonLib.h"

// LCD (change 0x27 to 0x3F if needed)

LiquidCrystal_I2C lcd(0x27, 16, 2);

// WiFi & Blynk

char auth[] = BLYNK_AUTH_TOKEN;

char ssid[] = "dileep";

char pass[] = "dileep1234567890";

// Energy monitor

EnergyMonitor emon;

// Relay pin

#define RELAY_PIN D6   // GPIO12

// Constants

float assumedVoltage = 220.0;

float costPerKwh = 5.5;

// Variables

float current = 0;

float power = 0;

float energy = 0;

float bill = 0;

int limitMode = 1;

float powerLimit = 50.0;

float energyLimit = 50.0;

BlynkTimer timer;

/* ---------- Blynk Inputs ---------- */

BLYNK_WRITE(V4) { powerLimit = param.asFloat(); }

BLYNK_WRITE(V5) { energyLimit = param.asFloat(); }

BLYNK_WRITE(V6) { limitMode = param.asInt(); }

/* ---------- Main Function ---------- */

void sendData()

{

current = emon.calcIrms(1480);          // Read RMS current

power = assumedVoltage * current;       // Power calculation

energy += power / 3600000.0;            // kWh

bill = energy * costPerKwh;

// Relay control (ACTIVE-LOW RELAY)

if (limitMode == 1)

{

if (power >= powerLimit || energy >= energyLimit)

digitalWrite(RELAY_PIN, HIGH);  // OFF

else

digitalWrite(RELAY_PIN, LOW);   // ON

}

// LCD Display

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("I:");

lcd.print(current, 2);

lcd.print("A ");

lcd.print("P:");

lcd.print(power, 1);

lcd.setCursor(0, 1);

lcd.print("E:");

lcd.print(energy, 3);

lcd.print("kWh");

// Blynk Output

Blynk.virtualWrite(V1, current);

Blynk.virtualWrite(V2, power);

Blynk.virtualWrite(V3, energy);

Blynk.virtualWrite(V7, bill);

}

/* ---------- Setup ---------- */

void setup()

{

Serial.begin(9600);

// I2C pins for ESP8266

Wire.begin(D2, D1);   // SDA, SCL

lcd.init();

lcd.backlight();

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, HIGH); // Relay OFF at startup

Blynk.begin(auth, ssid, pass);

// ACS712 (30A module)

emon.current(A0, 111.1);

timer.setInterval(1000L, sendData);

}

/* ---------- Loop ---------- */

void loop()

{

Blynk.run();

timer.run();

}